Dehumidifier

Thank you for selecting GREE Dehumidifier.

For proper operation, please read and keep this manual carefully.

If you have lost the Owner’s Manual, please contact the local agent or visit

www.gree.com or sent email to global@gree.com.cn for electronic version.

R-9116

Content

Operation Notices

Operation Area.....................................................................................1

Safety Precautions...............................................................................1

Parts Name...........................................................................................3

Operation Instructions

Operation Method.................................................................................4

Drainage Option....................................................................................6

Maintenance

Clean and Maintenance.......................................................................7

Malfunction

Troubleshooting................................ ..................................................8

Long term storage................................................................................9

Electric schematic diagram..................................................................9

Specifications.....................................................................................10

This appliance is not intended for use by persons (including children)

with reduced physical, sensory or mental capabilities or lack of experience

and knowledge, unless they have been given supervision or instruction

concerning use of the appliance by a person responsible for their safety.

Children should be supervised to ensure they are away from the appliance.

Do not dispose this product as unsorted municipal waste. Collec-

tion of such waste separately for special treatment is necessary.

(Graphics in this manual are only for reference.

Please refer to actual products for specific details.)

Operation Area

This dehumidifier is intended  for indoor residential

applications only. It should  not be used for

commercial or industrial  applications.

Correct

Wrong

Place the dehumidifier  on a smooth and level floor.

A dehumidifier operating  indoors will have no

effect in drying an adjacent  enclosed area,

such as a closet.

Above

5

Below

5

Place the dehumidifier  in an area where the

temperature will not below  5℃ or above 35℃.

Allow at least 40 inches  of air space on all

sides of the unit.

Close all doors, windows  and other outside

openings of the room  to improve the working effect.

Please keep the air inlet/outlet  clean and not blocked.

Do not use a dehumidifier  in a bathroom.

Avoid direct sunlight.

Note:

The dehumidifier has  rollers to aid placement. Do not attempt  to roll the

dehumidifier on carpet  or over objects. Otherwise, water may  spill out from

the bucket or the dehumidifier  may get stuck by the objects.

Safety Precautions

This appliance can be used by children aged from 8 years and

above and persons with reduced physical, sensory ormental

capabilities or lack of experience and knowledge if they have

been given supervision or instruction concerning use of the app-

liance in a safe way and understand the hazards involved.

Children shall not play with the appliance.

Cleaning and user maintenance shall not be made by children

without supervision.

Before operation, please check the power cord if it complies with the

requirement indicated on the nameplate.

Before cleaning, please turn it off and unplug the dehumidifier.

Make sure the power cord is not pressed by any hard object.

Do not remove the power plug or move unit by pulling the power cord.

Do not use any heating application near the dehumidifier.

Do not remove the power plug in wet hands.

Please use grounded power cord and make sure it's well connected and not

damaged.

- 1 -

Safety Precautions

Children and disabled people are not allowed to use the dehumidifier without

supervision.

Keep children from playing or climbing on the dehumidifier.

Do not place the dehumidifier under dripping objects.

Memory function is included in this dehumidifier. When nobody is taking care

of the unit, please turn it off and remove the power plug or disconnect power.

The appliance shall not be installed in the laundry.

The appliance must be positioned so that the plug is accessible.

The appliance shall be installed in accordance with national wiring regulations.

Do not repair or disassemble the unit by yourself.

Damaged power cord must be replaced by qualified technicians or manufacturer.

If abnormal condition occurs (e.g. burned smell), please disconnect power at

once and then contact local dealer.

Dehumidifier can not be disposed of everywhere. It you want to get rid of this

dehumidifier, please check with local handling disposal or information service

center about what to do.

- 2 -

Parts Name

Front Side

Control Panel

Front Grille

Water Bucket

Rollers

Back Side

Air Filter

Power Plug

- 3 -

Operation Method

Bucket full light indicates

bucket needs to be emptied

or it is not in the proper place.

4 hour or 2 hour

delay on/off

High，Mid and Low fan

indicator lights

Continuous operation on

indicator light

indicator lights

Power On

indicator light

Clean the Filter

indicator light

Notes:

Water bucket must be correctly  installed for the dehumidifier to operate.

Do not remove the bucket while  unit is in operation.

If you want to use drain hose to  drain water away, please install the hose

according to section "Drainage  method".

Each time pressing the effective  button on the control panel will give out

a "beep" sound.

When power is connected, power  indicator on the control panel will be on

and dehumidifier gives out a "beep"  sound simultaneously.

Basic Functions of the Buttons

ON/OFF  Pad

1

Press to turn the dehumidifier  on and off.

2

3

FILTER Pad

After 250 hours of operation, the  Clean the Filter indicator light will glow to

remind you toclean the filter. Remove  the filter and clean it. Press to turn

off the Clean the Filter light. See  the Care and Cleaning section.

Humidity Set ControlPads

The humidity level can be set within  a range of 80% RH (Relative Humidity)

to 35% RH (Relative Humidity)  in 5% reduce or at CO for continuous

operation.

NOTE: If CO (Continuous) is selected,  the dehumidifier will operate

continuously at its maximum dehumidification  settings if attached

to a hose to drain or until the bucket  is full.

For drier air, press the - pad and  set to a lower percent value (%).For

damper air, press the + pad and  set to a higher percent value (%).When

you first use the dehumidifier,  set the humidity control to 45% or 50%.

Allowat least 24 hours for the dehumidifier  toachieve the humidity level.

If you still havedamper air than  desired, set the humiditylevel to a lower

setting or select Continuousfor  maximum dehumidification.

- 4 -

This unit has 3-min lag due to  the device for protecting circuit .In order to prolong

the compressor's working life,  the compressor  will not start until the unit has run

for 3 minutes.

4   Display

Shows the set % humidity level  while setting, then shows the actual (+/– 5%

accuracy) room % humidity level.

5   FAN  SPEED Pad

Controls the fan speed.

Press to select High or Mid or  Low fan  speed.

Set the fan control to High for  maximum moisture removal. When the humidity

has been reduced and quiet operation  is preferred, set the fan control to Mid

or Low.

6   TIMER Pad

If unit is turned on and running  in timer mode for 2hr or 4hr,can turn off the unit.

When unit stand by and running  in timer mode for 2hr or 4hr can turn on the unit.

Other Instructions

11   BUCKET FULL  Light

Glows when the bucket is ready  to be emptied,or when the bucket is removed

or not replacedin the proper position.

12   Alarm

If the bucket is full or missing for  more than three minutes, an alarm will sound

for about 10 seconds to remind  you to empty and replace the bucket.

Auto Shut Off

3

The Water Level Control Switch  shuts off the dehumidifier when the bucket is

full, or when the bucket is removed  or not replaced in the proper position.

Auto Defrost

4

When frost builds up on the evaporator  coils,the compressor will cycle off and

the fan willcontinue to run until  the frost disappears.

5

Power Outage

In the case of a power outage  or interruption,the unit will automatically restart,

in the settings last used, after  the power is restored.

- 5 -

Drainage Option

Option 1 Emptying Manually

1. When the bucket is removed,the unit will shut off. Empty and replace the bucket.

Important:  Do not use the hose if using the water tank to collect water. When the

hose is connected, water will be drained out through it onto floor instead

of into the tank.

NOTES:

Do not remove the bucket while the unit is in operation or it has just stopped.

Otherwise it may cause some water to drip from the draining pan.

The bucket must be in place and securely seated for the dehumidifier to operate.

Option 2 Gravity Drain Hose

1. Water will automatically flow into a floor drain by attaching a 14mm inner diameter

hose (not included).

1) Open the drain cover on the side of the unit.

2) Pull out the rubber stopper on drainage joint.

3) Connect the hose to the drain tie-in. and lead it to the floor drain.

Note: Please keep the drain rubber plug carefully.To prevent the water from

flowing out of the water tank, make sure that the rubber plug has been firmly

inserted into the drain joint before starting up the unit.

Water Level

1.The dehumidifier will turn off automatically when the bucket is full, or when the bucket isremoved or not

replaced in the proper position.

2.The bucket must be replaced in the proper position forthe dehumidifier to work. The light will be on and

the dehumidifier will not run if the bucket is not in the proper position.

- 6 -

Clean and Maintenance

Warning:

Before cleaning, turn off the dehumidifier and disconnect power. Otherwise it

may lead to electric shock.

Do not wash the dehumidifier with water, or it may lead to electric shock.

Do not use volatile liquid(such as thinner or gasoline) to clean dehumidifier.

Otherwise it will damage unit's appearance.

1. Grille and Case

Grille

To clean the case:

When there is dust on the case, use soft towel

to dust it off; When the case is very dirty(greasy),

use mild detergent to clean it.

To clean the grille:Use a dust catcher or brush.

2. Water Bucket

Every few weeks, clean the bucket to prevent growth of mold, mildew and

bacteria. Use soft brush to clean the bucket and then rinse it.

3. Air Filter

Dehumidifier will remind you to clean the

air filter when it has been operating for

250 hours. If it is used in dusty places,

clean the air filter more often.

Buttons

To remove: 1.Press both sides of air-in grille,

pull it along the arrow direction

and then pull the air-in grille

upwards.

2.Pull the filter from the middle

clasps, bend the filter to let it

separate from upper clasps and

then remove the filter.

To clean: Clean the filter in warm, soapy

water. Rinse it and let the filter

dry before replacing it.

Warning:

Do not operate the dehumidifier without a filter. Otherwise the evaporator

will catch dust and affect unit's performance.

Do not dry the air filter with fire or electric hair dryer. Otherwise the air filter

may be unshaped or caught on fire.

- 7 -

Troubleshooting

Problem

Possible Causes

What To Do

Dehumidifier

does not start

The dehumidifier

is unplugged.

• Make sure the dehumidifier’s plug is pushed completely

into the outlet.

Dehumidifier  has reached its

preset level  or bucket is full.

• The dehumidifier  automatically turns  off when either

condition occurs.  Change to  a lower setting  or empty

the bucket  and replace properly.

Bucket is  not in the

proper position.

• The bucket must be in place and securely seated for

the dehumidifier to operate.

Power failure.

• There is a protective time delay (up to 3 minutes) to

prevent tripping of the compressor overload. For this

reason, the unit may not start normal dehumidifying for

3 minutes after it is turned back on.

Dehumidifier does not

dry the air as it should

Did not allow enough time

to remove the moisture.

• When first installed, allow at least 24 hours to maintain

the desired dryness.

Airflow is restricted.

• Make sure there are no curtains, blinds or furniture

blocking the front or back of the dehumidifier. See the

Choosing a Location section.

Dirty filter.

• See the Care and Cleaning section.

The Humidity Control may

not be set low enough.

• For drier air, press the “ - ” pad and set to a lower

percent value (%) or at CO (continuous) for maximum

dehumidification.

Doors and windows may not

be closed tightly.

• Check all doors, windows and other openings are

securely closed.

Clothes dryer may be

blowing moisture-laden

air into the room.

• Install the dehumidifier away from the dryer.

The dryer should be vented to the outside.

Room temperature

is too low.

• Moisture removal  is best at higher  room

temperatures.  Lower room temperatures  will reduce

the moisture  removal rate. Models  are designed  to

be operated  at temperatures  above 5°C.

Dehumidifier  runs

too much

Area to be  dehumidified

is too large.

• The capacity of your dehumidifier may not be adequate.

Doors and windows are open.

• Close all doors and windows to the outside.

Frost appears  on

the coils

Dehumidifier has been recently•     This is normal. Frost will usually disappear within 60

turned on, or room temperature

is below 5°C.

minutes. See the Automatic Defrost  section.

Controls cannot  be

set

The compressor  will turn  off

and the fan  will automatically

continue to  run for

• This is normal. Wait approximately 3 minutes and set

the fan as desired.

approximately  3 minutes at a

fixed setting.

Fan noise

There is sound created when

wind passing the unit. That is

the sound of wind.

• This is normal.

Water on floor

Hose connection  may be loose.•   Check the hose connection. See the  Removing

Collected Water  section.

Intend to use the bucket to

collect water, but a hose

is attached.

• Disconnect the hose if using the bucket to collect water.

See the Removing  Collected  Water  section.

- 8 -

Long term storage

When you do not use the unit for a long period, we suggest you to take following

steps to store the unit in a good condition:

Make sure there is no water left in the water tank and take off the draining hose

Take off the power cord and plug and warp them well.

Clean the unit and packed it well to keep it from dust.

IMPORTANT INFORMATION FOR CORRECT DISPOSAL OF THE PRODUCT IN ACCORDANCE WITH EC DIRECTIVE 2002/96/EC.

At  the end of its working life, the product must not be disposed of as urban waste.

It must be taken to a special local authority differentiated waste collection centre or to a dealer providing this service.

Disposing of a household appliance separately avoids possible negative consequences for the environment and health, deriving form inappropriate disposal and enables the constituent materials to be recovered to obtain significant savings in energy and resources. As a reminder of the need to dispose of household appliances separately, the product is marked with a crossed-out wheeled dustbin.