Multi High Performance Series Blender

INSTRUCTION MANUAL R-585

__________________________________________________

For your always safety and smoothly operation of this product, always

read this instruction manual carefully before using.

Attention

- Children being supervised not to play with the appliance.

- Be careful when handing the sharp cutting blades, emptying the bowl and during cleaning.

- Switch off the appliance and disconnect from supply before changing accessories or approaching parts that move in use.

- This appliance is intended to be used in household and similar applications.

- If the supply cord is damaged, it must be replaced by the manufacturer, its service agent or similarly qualified persons in order to avoid a hazard.

- The appliance is not to be used by persons (including children) with reduced physical, sensory or mental capabilities, or lack of experience and knowledge, unless they have been given supervision or instruction.

- This appliance can be used by children aged from 8 years and above and persons with reduced physical, sensory or mental capabilities or lack of experience and knowledge if they have been given supervision or instruction concerning use of the appliance in a safe way and understand the hazards involved.

- Children shall not play with the appliance.

- Cleaning and user maintenance shall not be made by children without supervision.

- Always disconnect the appliance from the supply if it is left unattended and before assembling, disassembling or cleaning.

- This appliance shall not be used by children. Keep the appliance and its cord out of reach of children.

- Appliances can be used by persons with reduced physical, sensory or mental capabilities or lack of experience and knowledge if they have been given supervision or instruction concerning use of the appliance in a safe way and if they understand the hazards involved.

For your always safety and smoothly operation of this product, always read this instruction manual carefully before using.

Safety Warning

Keep this manual for future reference.

- DO NOT put motor base in water or any other liquids due to risk of electrical shock.

- Appliance is not intended for use by persons with any reduced physical, sensory of mental capabilities (including children). Ensure close adult supervision necessary when appliance is used by or near children.

- Never touch or make contact with moving parts, especially the blades, as it may cause serious injuries.

- Unplug power cord from outlet when appliance is not in use, before disassembling, when cleaning or changing parts.

- DO NOT operate appliance with a damaged cord or plug, if the unit malfunctions, or has been damaged in any manner.

- The machine must be plugged in a power supply that matches the appliance’s power requirements. Please consult a qualified electrician for proper electrical needs.

- DO NOT use attachments not recommended for this product, by doing so, all warranty with be voided. Alterations or modifications is not commended and may result in bodily injury.

- DO NOT use outdoors.

- DO NOT place machine on or near hot gas or electrical burner, heated oven, or allow machine to touch hot surfaces. Any external heat source can damage the machine.

- Keep hands and utensils out of the blender when motor is running to prevent any possibilities or severe personal injury or damage to the appliance. A rubber scraper may be used, but must be used only when the blender is not running.

- Handle Blades with care. As blades are very sharp, never try to remove the blades while the container is on the motor base.

- DO NOT operate with loose or damaged blades-replace immediately.

- The blender needs to rest 5 minute after continuous working 5 minutes.

- NEVER place blade assembly on motor base without blender container properly attached.

- The maximum capacity is equivalent to the liters listed on the container, and is less with thicker mixtures.

- When making nut butters or oil based foods, DO NOT blend for more than one minute after mixture starts circulation. This could lead to dangerous overheating.

- See Care & Cleaning Instructions for all blenders. NOT FOR WATERJET CLEANING.

- The appliance must be positioned so that the plug is accessible.

- DO NOT use carbonated liquids in the blender jar or any blending container. Using carbonated liquids in the blender jar or any blending container may result in injury.

- DO NOT blend oil or oil-based products such as peanut butter for greater than 5 minutes.

- Always operate blender with cover in place.

- This machine should be working 3 minutes rest one minute.

- This machine only could be use with cold water and its temperature not over than 40 degree.

WARNING: Appliance should not be cleaned with a water jet.

About Thermal Protection

- The built-in thermal protector keeps track of the motor temperature and will warn the user of a heating malfunction. The protector will eventually shut the motor down. A display will instruct the user to remove the container from the motor until sufficiently cooled. This process takes approximately 2 minutes.

- If your recipe is causing a repeated heating problem, your recipe may either be too thick or too many contents in the machine. Consider adding more liquid or other means.

Introduction

Congratulations on the purchase of your Multi-High Performance Series Blender.

Your new commercial grade blender will allow you to process your drinks and ingredients quicker than ever before.

The heavy duty power motor allows blades to spin at 26,000 －30,000 rpm,allowing you to blend anything from fruits and vegetables to coffee beans,ice and more. By crushing ice,you can make the tastiest smoothies and juices in seconds and there is no more sipping and chewing,the power of the motor means every drink will be blended to a smooth texture.

Variable Speed Knob Control Panel

1. Before starting, make sure that the blender is unplugged and container

is not positioned on the blender .

2. Put ingredients into container and cover it. Do not fill above MAX line.

3. With the unit in “OFF” position, place container on blender base. Be sure

container is fully seated and that drive coupling is properly engaged.

Plug blender cord into outlet.

4. This blender has variable speed controls. Set blender to desired speed from Low to High.

5. The left paddle switch defaults to the VARIABLE setting in order to use the speed adjustment option. Press right paddle switch up to the “o” position for pulse function.

6. When finished blending, set paddle switch to “OFF” position.

7. This blender is equipped with a “PULSE” option. Hold left paddle switch in down position to engage “PULSE” function. Unit will run on max speed (30,000 RPM) until the paddle switch is released.

8. Allow motor to come to a complete stop before removing container from blender base. Never place the container on the motor base or remove it from the motor base while motor is running.

9. Always hold container with one hand when starting and while blending. Never operate without jar lid.

10. Wash and dry container after each use. See Cleaning & Care Instructions.

Electronic Touchpad Control Panel With Timer

- Before starting, make sure that the blender is unplugged and container is not positioned on the blender.

- Put ingredients into container and cover it. Do not fill above MAX line.

- Place container on blender base. Be sure container is fully seated and that drive coupling is properly engaged. Plug blender cord into outlet.

This blender is equipped with an electronic membrane keypad. The ON/OFF and speed,Timerbuttons and a 999-second countdown timer with up and down arrows and confirm with ok  to set desired speed and time.

- Press the ON/OFF button to turn on the blender. Red LED will illuminate. The electronic timer will indicate 180 seconds (default) . Press the up  or down  arrow to set the desired time. The electronic timer will indicate run time in 1-second (short press) or 10-second(long press) intervals up to maximum of 999 seconds. After the desired time is set, press speed from 1 to 8 gears of speed as your need, then press ok to start the blender.

The timer will count down to 0 in 1-second intervals and blender will shut off. Pressing the ON/OFF button will override the timer and the blender will stop regardless of time left on timer.

NOTE: If you want to increase or decrease time while timer is counting down, press Timer and up

or down arrows. Time will increase or decrease in 3-second intervals.

- If the blender is not used for three seconds, the timer function will turn off and return to manual mode. To reactivate timer, press Timer,then up or downarrows.  Time will increase or decrease in 3-second intervals.

For speed adjust operation, press speed  from 1 to 8 gears speed. When pressing speed, the LED screen will show “1”, which indicates the lowest speed, and press up  or down  arrows. Speed will increase from 1 to 8 gears, or decrease from 8 to 1 gears.  When finished blending, press ON/OFF. For momentary operation, press the PULSE  button. Motor will run on highest speed until the button is released.

- NOTE: The Electronic Blender is with slowstart feature, when pressing ON/OFF, and ok, the unit will start in the lowest speed for 3 seconds and then switch to highest speed, the default time is 180 seconds when not using Speed  and Timer button.

- Allow to remove the jar from the motor base while motor is running, the switch will shut off automatically, put the jar back to motor base until the drive stop moving completely.

Wash and dry container after each use. See Care & Cleaning instructions.

- To turn the unit off, press ON/OFF button. Red LED light will go off.

Reprogrammable Settings for Knob Control

- Before starting, make sure that the blender is unplugged and container is not positioned on the blender.

- Put ingredients into container first, solid items last. Thawed or partially thawed fruit is recommended, do not fill above MAX line, and cover it.

- With the unit in OFF position, place container on blender base. Be sure container is fully seated and that drive coupling is properly engaged. Plug blender cord into outlet.

- This blender has variable speed Knob controls. Set blender to desired speed between 1 and 10.

- High speed is recommended for heavy-duty blending. You may also want to break the food into smaller pieces for quicker processing.

- For blenders with variable speed Knob settings, select the desired speed. Most foods should be blended on HIGH speed.

- Start heavy or thick foods on LOW speed and then switch to a HIGH speed once mixture beings processing.

- Start on LOW if the blender is full to the maximum line or if you have placed hot liquids or food in the blender.

- If your mixture stops circulating, you have likely trapped an air bubble. You can either use the mixing rod through the lid or stop the motor and remove the container and stir the mixture from the sides of the container using a rubber spatula.

Note: Do not use metal utensils as it may damage the container.

- After the blender has been stopped, let the blade come to a complete stop before removing the lid and/or container from the motor base.

Warning: If you detect any change in the sound of the machine, or a distinct hard object comes into contact with the blades, do not serve the drink. Inspect your blade assembly for loose or nicked parts.

- This blender is equipped with a PULSE function. Unit will run on max speed (30,000 RPM) until the paddle switch is released.

- Allow motor to come to a complete stop before removing container from blender base. Never place the container on the motor base of remove it from the motor base while motor is running.

- Always hold container with one hand when starting and while blending. Never operate without jar lid.

Wash and dry container after each use. See Care & Cleaning instructions.

- Turn power OFF and unplug the machine when left unattended.

Reprogrammable Settings for Electronic Touchpad Control with Timer

1. Before starting, make sure that the blender is unplugged and container is not positioned on the blender.

2. Put ingredients into container first, solid items last. Thawed or partially thawed fruit is recommended, do not fill above MAX line, and cover it.

3. With the unit in OFF position, place container on blender base. Be sure container is fully seated and that drive coupling is properly engaged. Plug blender cord into outlet.

4. The motor speed can be adjusted by pressing speed, and press up  or down  arrows buttons. Speed ranges from one (lowest) to eight (highest) speed reprogrammable beverage stations. Speed ranges from 15% to 100%.

- The Speed Up  will increase the motor speed display in 1% increments and the Speed Down  button will decrease the motor speed display in 1% increments. Holding down either button will quickly scroll through speeds.

- Using the Speed with Up  and down  arrows, then ok button, set desired motor speed.

- If no speed is selected, motor will default to 100%.

- NOTE: The motor does not run at the newly set speed until the ON/OFF button is pressed. If  Speedbutton is not pressed within 3 seconds, the new speed setting will be stored for next use and the blender will return to STANDBY mode;

- Press ON/OFF button, after set desired motor speed and time, press OK  to run the blender. The LED will display the time count backwards. Default time is 180 seconds, and the default speed is the highest speed without setting the time and speed. Motor speed and time can be changed while blender is running by simply pressing Speed and Time, then arrows to  or button.

- To stop continuous blending operation, press ON/OFF button.

Resetting Thermal Protection

  - The blender you have purchased is equipped with an Automatic Reset Switch to protect the motor from overheating.

  - If your blender stops running under heavy use, turn the power switch to the ON/OFF position and unplug the power cord. Empty the blender container of all its contents and allow approximately 5 minutes for the motor to cool down.

  - Plug the power cord back into the outlet and continue blending.

Hot Blending Instructions

To minimize the possibility of being scalded when blending hot materials, the following precautions and procedure must be observed.

- Hot materials are those that are 100℉ (38℃) or hotter.

- Limit the amount of materials processed to 1 pint.

- Completely remove the center portion of the two-piece container lid before starting motor (this will allow venting of the rapidly heated and expanded air inside the container when blender is running).

- Always start blending on lowest speed setting, and the progress to desired speed.

- Never stand directly over blender when it is being started or while it is running.

TURNING OFF YOUR BLENDER

Whether your blender is in STANDBY mode, NEW PROGRAM mode, or the motor is running, pressing the ON/OFF button will stop the unit from running and will turn the power off. The LED display will turn off.

NOTE: All programs and current settings will be restored next time you turn unit on.

Care & Cleaning

THE CONTAINER MUST BE CLEANED AND DRIED AFTER EACH USE OF THE BLENDER ACCORDING TO THE FOLLOWING INSTRUCTIONS. FAILURE TO DO SO MAY ALLOW RESIDUES OF PROCESSED MATERIAL TO COLLECT ON THE INTERNAL SEALS OF THE BLADE ASSEMBLY, RENDERING THEM INEFFECTIVE.

Before Cleaning

- Disconnect the power plug before cleaning.

- Be careful when washing the container; keep your hands away from the blades.

- DO NOT allow contact with water with the motor base.

Cleaning the Container

- Fill container with 1/4 full of warm water and a couple drops of liquid dishwashing detergent in. Place container to motor base and firmly put the lid onto the machine and operate the machine for 30 seconds. Proceed to empty the container and repeat if necessary.

- Rinse the container with 3/4 full with warm water. Return the container to motor base and firmly put the lid onto the container. Run machine for 30 seconds and empty the container.

- If any residue remains, remove the blade assembly and wash all container parts in warm soapy water. Reassemble the blade assembly for next use.

- Allow container to air dry.

Cleaning the Lid

- Separate the lid from the container. Wash in warm, soapy water.

- Rinse clean under running water and dry.

- Reassemble before next use.

Cleaning the Motor Base

- Unplug the power cord.

- Use a damp soft cotton cloth to wash the outside surface only.

NEVER IMMERSE MOTOR BASE IN WATER OR OTHER LIQUIDS.

- Clean around the edges of the switches, knob and button to ensure they work freely. Work the switches back and forth a couple of times.

- Dry with a soft cotton cloth.

Any other servicing should be performed by an authorized service representative.

| Troubleshooting |  |
| --- | --- |
| Problem | Solution |
| - Higher than normal Levels of vibration and Container noise | - Check blade assembly for loose, damaged or nicked components and replace. |
| - Leaking from the bottom of the container |  |
| - Machine will not run | - Double-check to make sure power cord is firm plugged into the power outlet. - Check to make sure switches are clean and moving freely. - The thermal protector may have shut the machine off. Leave power off for up to 45 minutes. Try circulating air with a fan at the bottom of the motor base. |
| - Loud noise from blade area | - Drive socket may be broken. Check for a fine crack down the center or worn teeth. Replace with a new drive. |
| - Blades do not spin or rotate worn |  |

If you can not fix your machine with these troubleshooting suggestions, please have the professional fixed up available.

Follow these steps if the unit fails to operate:

- Check instructions have been follow closely and correctly

- Check “Troubleshooting Guide” for possible solutions

- Check fuse has been blown.

IMPORTANT INFORMATION FOR CORRECT DISPOSAL OF THE PRODUCT IN ACCORDANCE WITH EC DIRECTIVE 2002/96/EC.

At  the end of its working life, the product must not be disposed of as urban waste.

It must be taken to a special local authority differentiated waste collection centre or to a dealer providing this service.

Disposing of a household appliance separately avoids possible negative consequences for the environment and health, deriving form inappropriate disposal and enables the constituent materials to be recovered to obtain significant savings in energy and resources. As a reminder of the need to dispose of household appliances separately, the product is marked with a crossed-out wheeled dustbin.