Technaxx®

FMT600BT Transmitter

User Manual

Transfer of music and calls directly from your Bluetooth device

to your car radio

- This device complies with the requirements of the standards referred to the Directive R&TTE 1999/5/EC. The Declaration of Conformity you find here: www.technaxx.de/ (in bar at the bottom “Konformitätserklärung”). Before using the device the first time, read the user manual carefully.

Turn OFF nearby Bluetooth devices to avoid signal interference!

Service phone No. for technical support: 01805 012643 (14 cent/minute from German fixed-line

and 42 cent/minute from mobile networks). Free Email: support@technaxx.de

Features

 Wirelessly transmit phone calls from Bluetooth mobile phones to the car FM stereo system, for every car type without modification  Take convenient phone calls and still drive safely and transmit wirelessly your MP3 music to the car radio via UKW  Built-in Microphone for hands-free handling (switchs to hands-free mode automatically from music playing status when receiving calls)  USB port for charging a mobile device, e.g. iPad (Output 5V/2.1A)  USB port to plug in an USB flash drive (up to 32GB) with MP3 music tracks  Bluetooth chip is compatible with a wide range of Bluetooth enabled devices (= Pairing)  Last saved UKW frequency stays saved  Steaming music in MP3/WMA format from a USB flash drive  3.5mm AUX jack: music playback from external audio devices (iPod, MP3 player, etc.)  Remote control (included) for more convenient operation  “Voice prompts” (means: spoken announcement)  Easy handling and installation (just plug it into the 12V/24V socket of a vehicle)

Technical Specifications

| Bluetooth module / BT version | CSR BC8635 / Bluetooth version 4.0 |
| --- | --- |
| Bluetooth operating distance | max. 10m |
| LCD display size | 20 x 25.5 mm (1.1“) |
| Frequency range | 87.6 – 107.9 MHz, max. 3m |
| Frequency stability | ±10ppml |
| Audio input | 3.5mm stereo jack |
| Music format | MP3, WMA |
| Memory | USB up to 32GB |
| FM transmitter input | DC 12V – 24V |
| Charger output | 5V / 2.1A |
| Weight / Dimensions | 45 g / (L) 7,5 x (W) 3,4 x (H) 10,5 cm |
| Package Contents | FMT600BT Transmitter, AUX–IN cable, Remote control, User Manual |

View of device

AUX-IN port

SD Card Slot

Volume + / Volume –

USB Reader

Previous button / Next button

Frequency modulation “M”

Output 5V / 2.1 A

Answer / Hang off Play / Pause

Using FM Transmitter function

(a) Plug the FM transmitter into the cigarette lighter port or power port of your vehicle.

(b) First tune your radio to a free FM frequency, then match the same frequency to the FM transmitter.

(c) Insert a USB flash drive, which contains MP3 files, into the USB port. It will play and transmit the MP3 files automatically. Attention: only the inward USB Port can serve as a USB Reader!

(d) Line-IN function: Use the provided audio cable to connect a MP3/MP4 player or another external device to the AUX–IN port of this FM transmitter. When using this function, you have to stop playback from the USB flash drive, and all functions will be controlled by the external device.

Using Bluetooth function

Using Bluetooth for the first time, you need to pair your mobile device with this FM transmitter.

(a) Activate the Bluetooth function on your mobile device. Then search the NEW device. When the mobile device has detected this FM transmitter with the name "TECHNAXX", pair the devices by using the original password "0000".

(b) In the music playback mode this FM transmitter will automatically switch to the telephone mode, when there is an incoming call.

Bluetooth calling

| Answer the incoming call | Press the blue knob |
| --- | --- |
| Hang off a call | Press the blue knob |
| Dial the last number | Press 2x the blue knob |
| When making a call, you can only dial the phone number by your mobile phone, but once the call is connected, you can use the hands-free function. |  |

Using USB charger function

When this FM transmitter is powered from car lighter socket, connect the USB output port 5V/2.1A of this FM transmitter to your mobile device with a special USB cable** (**not included). This FM transmitter can be used as charger for mobile devices. ATTENTION: The USB output port with 5V/2.1A is for charging an external device (e.g. iPad).

Get access to Menu Lists

(a) “Play” mode

While music is playing, press the “M” button for more then 3 seconds to see this menu:

| Language | select the language you prefer |
| --- | --- |
| Contrast | change the contrast of the display |
| Firmware version | see the firmware version of the FM transmitter |
| Tx-Power | change the transmission power depending on the region (Europe, ROW) |
| Exit | select to finish the configuration |

(b) “Pause” mode:

Music will pause when you press the button. Then press the “M” button for more then 3 seconds to see this menu list:

| U-disk Dir | select the file to play |
| --- | --- |
| Delete file | select the file to delete |
| Delete all | select to delete all the file |
| Language | select the language you prefer |
| Contrast | change the contrast of the display |
| Firmware version | see the firmware version of the FM transmitter |
| Exit | select to finish the configuration |

ATTENTION (see remote control)

- Conversion between regional configurations

This product also supports various regional configurations. Press the “M” button for more then 3 seconds till the menu appears. Choose “Tx-Power” and then press the “M” button on the FM transmitter to choose “Europe” (in Europe) or “ROW” ( in the rest of the world). Then press the “M” button on the FM transmitter to confirm. [The sound quality under “ROW” is better than under “Europe” mode.]

- Conversion from Bluetooth to USB flash drive

When the FM transmitter is playing the music from your mobile phone via Bluetooth, press the “Pause” button on the FM transmitter. After ~ 10 seconds it appears on the screen the following: “Press M key”. Then press the “M” button. The FM transmitter then will play the music from USB flash drive.

View of remote control

Most of the function can be fulfilled by the remote control. Folder Switch

Loop mode Selector

EQ Selector

Number Selector

Answer / Hang off Play / Pause

Channel + / –

Volume + / –

Prev. / Next (Previous Song / Next Song)

Quick Using Guide

| Operation Guide | FM transmitter | Remote Control |
| --- | --- | --- |
| Answer / Hang up a call | Press button to answer a call Press button to hang off a call | Press button |
| Play music / Pause music | Press button to play music Pressbutton again to pause music playback | Press button |
| Increase the volume | Rotate clockwise to increase the volume. | Press VOL+ to increase the Volume. |
| Decrease the volume | Rotate anticlockwise to decrease the volume. | Press VOL– to decrease the volume. |
| Set Frequency | Press “M” button first. Then press to increase the frequency. Press to decrease the frequency. | Press CH+ button to increase by 0.1MHz. Press CH– button to decrease by 0.1MHz Or press the numbers of the frequency what you desired, then press CH+ button to confirm. |
| Choose Music | Press to play the next song. Press to play the previous song. | Press to play the next song. Press to play the previous song. Or press the number of the song and then press to confirm. |
| Choose EQ | Support by remote control only | Press EQ button to choose your favorite EQ. |
| Change Loop Mode | Support by remote control only | Press button to change to loop mode. |
| Change Language | Press “M” button for more then 3 seconds first. Then choose “Language” option by pressing and. Short press “Language” to choose the desired language by pressing and . Then press “M” to confirm. | Support by FM transmitter only |
| Change Contrast | Press “M” button for more then 3 seconds first. Then choose “Contrast” option by pressing and . Short press “Contrast” to choose a contrast setting by pressing and. Then press “M” to confirm. | Support by FM transmitter only |
| Switch Folder | Press “M” button under PAUSE MODE for more then 3 seconds first. Then choose “USB-disk Dir” or “Card Folder” and press “M” button to confirm. Choose your desired folder and then press “M” button to confirm this folder. | Press Folder+ or Folder– switch the folder. |

How to solve problems

The FM transmitter does not start: Check the fuse first. Exact handling see at the beginning of the user manual (How to replace the fuse).

The fuse thread is blown: The fuse has to be exchanged by an equal fuse. Exact handling see at the beginning of the user manual (How to replace the fuse).

The device does not switches on again: Check the function of the cigarette lighter.

The cigarette lighter has no function: Contact your car garage.

Problem when operating: First switch off the FM transmitter and then switch it on again.

Warnings

 Do not attempt to disassemble the device, it may result in short-circuit or even damage.

 Do not drop or shake the device, it may break internal circuit boards or mechanics.

 Keep the device in dry and ventilated environment. Avoid high humidity and high temperature. Also batteries should not be exposed to excessive heat or direct sunlight.

 Keep the surface of the device clean, but do not use organic solvents.

 Keep the device away from little children.

Hints for Environment Protection: Packages materials are raw materials and can be recycled. Do not disposal old devices or batteries into the domestic waste. Cleaning: Protect the device from contamination and pollution (use a clean drapery). Avoid using rough, coarse-grained materials or solvents/aggressive cleaner. Wipe the cleaned device accurately. Important Notice: Should battery fluid leak from a battery, wipe the battery case with a soft cloth dry. Distributor: Technaxx Deutschland GmbH & Co.KG, Kruppstr. 105, 60388 Frankfurt a.M., Germany